DSTAR GUIDE

For ICOM 9700

Table of Contents

What Is D-STAR?	3

Basic Setup	5

Add Repeaters	6

Add Repeater as a Channel	8

Adding a simplex Hotspot	10

Pictures	12

Appendix:	14

ODC Repeaters	14

What Is D-STAR?

D-STAR (Digital Smart Technologies for Amateur Radio) is a digital voice and data protocol developed specifically for amateur radio by the Japan Amateur Radio League (JARL). It allows licensed operators to communicate using digital voice and low-speed data over VHF, UHF, and microwave bands.

🔑 Key Features:

- Digital voice (DV) and digital data (DD) modes

- Internet linking via gateways

- Callsign-based routing

- Reflectors for group communication (like conference rooms)

🧠 How D-STAR Routing Works

D-STAR uses four key fields to route your signal:

| Field | Purpose |
| --- | --- |
| MYCALL | Your own callsign |
| URCALL | Who you're calling (e.g., CQCQCQ for general) |
| RPT1 | The repeater you're transmitting into (with port letter) |
| RPT2 | The repeater or gateway you're routing through (with port letter) |

Example for local repeater use:

- URCALL: CQCQCQ

- RPT1: VE3ABC C

- RPT2: VE3ABC G

- MYCALL: VA3XYZ

✍️ Nuances of Shorter Callsigns and Spacing

D-STAR fields are exactly 8 characters long. If your callsign is shorter than 7 characters, you must pad it with spaces so that the port letter (A, B, C, G, etc.) is in the 8th position

🧩 Padding Rules:

| Callsign Length | Example | Port Letter | Correct Format |
| --- | --- | --- | --- |
| 4 characters | W1AW | C | W1AW C |
| 5 characters | K9DOG | B | K9DOG B |
| 6 characters | VE3XYZ | G | VE3XYZ G |
| 7 characters | N0CALLS | A | N0CALLSA |

⚠️ Spaces are invisible, so it's easy to misplace the port letter. Always count carefully!

🛠️ Tips for Programming D-STAR Fields

- Use a programming app like Icom’s CS-9700 or RT Systems to visualize spacing.

- When entering callsigns manually, use the radio’s keypad to insert spaces before the port letter.

- Double-check that the 8th character is the port letter, not earlier.

Register

How to Register Your Callsign on the D-STAR Network (Canada)

✅ Why Register?

Registering your callsign is required to:

- Use reflectors (like XLX405D)

- Operate in Terminal Mode or Access Point Mode

- Use hotspots, DV Dongles, or network routing

🛠️ Step-by-Step Registration via BCFMCA

🔹 Step 1: Go to the Registration Portal

Visit: 🔗 https://dstar.bcfmca.bc.ca/Dstar.do

🔹 Step 2: Create an Account

- Click Register.

- Fill in:

  - Callsign (in ALL CAPS, no suffixes like /9700)

  - Name, email, and a password

- Read and accept the User Agreement

- Click OK to submit

📧 You’ll receive an email once your registration is approved. This may take a few hours to a couple of days.

🔹 Step 3: Register Your Terminal (Radio)

Once approved:

- Log in with your callsign and password

- Click Personal Information

- On the first blank line:

  - Check the box on the left

  - In the Initial box, press the space bar once (this creates a blank terminal ID)

  - Do NOT check the “AccessPoint” box

  - Click Update

🧠 This step is mandatory. Without it, your radio won’t be recognized by the D-STAR network.

🔹 Step 4: Program Your Radio

On your IC-9700 (or other D-STAR radio):

- Go to Menu > Set > My Station > My Call Sign

- Enter your registered callsign exactly (e.g., VA3XYZ)

- Do not include suffixes like /9700 in this field

🧪 Check Your Registration

Use the D-STAR registration checker: 🔍 https://regist.dstargateway.org/regcheck/index.php

Basic Setup

Here’s a simple guide to help you get started with D-STAR

📛 1. Set Your My Call Sign (DV Mode)

This is your registered D-STAR callsign, and it's used to identify your station on the network.

Steps:

- Go to: Menu > Set > My Station > My Call Sign

- Enter your callsign, e.g.:

  - VA3XYZ

  - or VA3XYZ /9700 (if you want to identify the radio model)

✅ Why use /9700? 
Adding /9700 helps others (and dashboards) know which radio you're using. It’s especially useful if you operate multiple D-STAR radios or hotspots.

⚠️ Note: Some systems may truncate or ignore characters beyond 8, so this is mostly cosmetic and for TX message or DD mode—not for RPT1/RPT2 fields.

💬 2. Set Your TX Message (DV Mode)

This message is transmitted with your voice and appears on other users’ screens.

Steps:

- Go to: Menu > Set > My Station > TX Message (DV)

- Enter up to 20 characters, e.g.:

  - VA3XYZ /9700

  - Chatham ON /9700

  - IC-9700 via XLX405D

📌 This is where /9700 is most useful—it’s visible to others during QSOs.

🌐 3. Set My Call Sign (DD Mode)

If you're using DD Mode (Digital Data over 1.2 GHz), you can set a separate callsign.

Steps:

- Go to: Menu > Set > My Station > My Call Sign (DD)

- Enter your callsign (e.g., VA3XYZ/9700)

🧠 This is used for identifying your station in DD mode, such as when using the IC-9700 as a network interface.

🧾 4. Terminal/AP Call Sign (for Terminal Mode)

When using Terminal Mode, you must set a unique 8-character callsign with a module letter.

Steps:

- Go to: Menu > Set > DV Gateway > Terminal/AP Settings

- Set Terminal/AP Call Sign to:

  - VA3XYZ T (7 characters + T)

  - You can optionally use VA3XYZ/9 or similar if it fits within 8 characters

⚠️ This field must be exactly 8 characters, with the module letter (T) in the 8th position.

✅ Summary Table

| Setting | Example Value | Purpose |
| --- | --- | --- |
| My Call Sign (DV) | VA3XYZ or VA3XYZ/9700 | Identifies your station on D-STAR |
| TX Message (DV) | IC-9700 Chatham | Transmitted with your voice |
| My Call Sign (DD) | VA3XYZ/9700 | Used in DD mode |
| Terminal/AP Call Sign | VA3XYZ T | Required for Terminal Mode |

# GPS Configuration

Here’s a complete guide to configuring so that DR Mode’s “Near Repeater” feature works properly. This allows your radio to automatically list nearby D-STAR repeaters based on your current location.

🧭 Why Enable GPS?

Enabling GPS on the IC-9700 allows you to:

- Use “Near Repeater” in DR Mode to find local D-STAR and FM repeaters

- Automatically update your position for D-PRS (D-STAR Position Reporting System)

- Log GPS coordinates with received signals or photos

🛠️ Step-by-Step: Configure GPS for DR Mode

🔹 Step 1: Connect a GPS Receiver

The IC-9700 does not have a built-in GPS, so you’ll need to connect an external GPS receiver.

Options:

- ICOM UX-241 GPS antenna (plugs into the GPS port)

- Any NMEA-compatible GPS with a 2.5mm plug (4800 or 9600 baud)

- GPSDO or GPS puck with serial output and proper voltage

Plug the GPS into the GPS port on the rear panel of the IC-9700.

🔹 Step 2: Enable GPS Input

- On the radio, go to: Menu > Set > GPS > GPS Select

- Choose: External GPS

- Then go to: Menu > Set > GPS > GPS Set

- Set:

  - GPS Auto Time: ON (optional)

  - GPS Indicator: ON (to show GPS lock on screen)

📌 Once the GPS locks onto satellites, a small satellite icon will appear on the top of the screen.

🔹 Step 3: Use “Near Repeater” in DR Mode

- Press the DR button to enter DR Mode.

- Tap the FROM field.

- Tap Near Repeater

- Choose:

  - ALL (for all repeaters)

  - DV (for D-STAR only)

  - FM (for analog repeaters)

The radio will now display a list of nearby repeaters sorted by distance!

✅ Tips for Best Results

- Make sure your GPS antenna has a clear view of the sky.

- Allow a few minutes for the GPS to acquire satellites after power-on.

- You can also manually enter your position under: Menu > Set > GPS > Manual Position if GPS is unavailable.

# GPS Configuration: Manual Position

Manually Set GPS Coordinates on the IC-9700

✅ Why Use Manual GPS?

- You don’t have a GPS antenna connected

- You want to simulate a location for testing

- You’re operating indoors or in a fixed location

🛠️ Step-by-Step Instructions

1. Open the GPS Settings Menu

- Press MENU

- Tap Set

- Tap GPS

- Tap GPS Select

- Choose Manual

2. Enter Your Coordinates

- Go back to the GPS menu

- Tap Manual Position

You’ll now see fields for:

- Latitude (e.g., 42° 24.000' N)

- Longitude (e.g., 82° 11.000' W)

- Altitude (optional)

3. Input Your Location

- Use the touchscreen keypad to enter your latitude and longitude

- Be sure to select the correct N/S and E/W directions

- Tap Enter or OK to save

📌 You can find your exact coordinates using a smartphone, GPS app, or Google Maps by right-clicking your location and copying the decimal or DMS format.

✅ Confirm It’s Working

- In DR Mode, tap FROM > Near Repeater

- The radio will now use your manual coordinates to list nearby repeaters

# Add Repeaters

Here’s a step-by-step guide to help you add new D-STAR repeaters using an SD card:

🧭 What You’ll Need

- A computer with internet access

- An SD card (formatted in the IC-9700)

- A card reader (if your PC doesn’t have an SD slot)

- The latest repeater list from Icom

📥 Step 1: Download the Repeater List

- Go to the Icom Support Page

- Search for your model: IC-9700

- Download the Repeater List Update ZIP file

- Extract the ZIP file on your computer

💾 Step 2: Copy to SD Card

- Insert the SD card into your PC

- Locate the extracted files:

  - If using a CSV file, copy it to the RptList folder on the SD card

  - If using an ICF file, copy it to the Setting folder

- Safely eject the SD card

📌 Tip: Format the SD card in the IC-9700 before use to ensure correct folder structure.
 DATA LOSS WARNING: Don’t do it again if you already formatted it previously.

📡 Step 3: Import into the IC-9700

- Insert the SD card into the IC-9700

- Go to Menu > SD Card > Load Settings

- Choose the appropriate file (CSV or ICF)

- Confirm and load the data

🔍 Step 4: Access New Repeaters

- Press and hold the DR button to enter DR mode

- Tap FROM > Repeater List

- Browse or search for your newly added repeaters

- Tap to select and operate

🧠 Bonus: Manually Add a Repeater

If you want to add a repeater manually:

- Go to Menu > Set > My Station > Repeater List

- Tap Add and enter:

  - Repeater Callsign

  - Frequency

  - Offset

  - RPT1 and RPT2 Callsigns

- Save and access it via DR mode

# Add a Repeater as a Channel

🧰 What You’ll Need

- Your D-STAR repeater’s:

  - Frequency

  - Offset

  - RPT1 and RPT2 callsigns

- Your own callsign entered in the radio (Menu > Set > My Station > My Callsign)

🔧 Step 1: Enter Memory Mode

- Press the MENU button.

- Tap Memory CH to enter memory channel mode.

- Choose an empty memory slot or tap MW (Memory Write) to create a new one.

📡 Step 2: Set Frequency and Offset

- Set the receive frequency of the repeater.

- Set the transmit frequency (based on the repeater offset).

- Choose DV mode (Digital Voice) by pressing the MODE button.

🏷️ Step 3: Set D-STAR Parameters

- Tap Menu > Set > DV Memory

- Enter the following:

  - URCALL: CQCQCQ (for general QSOs)

  - RPT1: Repeater callsign + module (e.g., VE3ABC C)

  - RPT2: Gateway callsign + module (e.g., VE3ABC G)

- Save the settings to the selected memory channel.

💡 Modules:

- C = VHF

- B = UHF

- A = 1.2 GHz

- G = Gateway (for internet linking)

💾 Step 4: Save to Memory

- Press and hold the MW button.

- Choose the memory slot.

- Confirm to save.

✅ Step 5: Use the Channel

- Switch to Memory Mode

- Select your programmed channel

- You're ready to transmit on D-STAR just like FM!

📘 Example: Programming VE3ABC on 145.670 MHz

| Parameter | Value |
| --- | --- |
| Frequency | 145.670 MHz |
| Offset | +600 kHz |
| Mode | DV (Digital Voice) |
| URCALL | CQCQCQ |
| RPT1 | VE3ABC C |
| RPT2 | VE3ABC G |

# Adding a simplex Hotspot

Here’s a step-by-step guide to adding a simplex D-STAR hotspot (like Pi-Star) to your IC-9700. This setup allows you to use your personal hotspot for D-STAR communication without relying on a repeater.

📡 What You’ll Need

- A Pi-Star-based hotspot (e.g., ZumSpot, MMDVM, etc.)

- Your hotspot frequency (typically 433.550 MHz or 145.670 MHz)

- Your callsign

- The Icom IC-9700 in DV mode

🛠️ Step 1: Configure Your Pi-Star Hotspot

- Power on your Pi-Star device.

- Connect to it via a browser at http://pi-star.local or its IP address.

- Go to Configuration > General Configuration:

  - Callsign: Your callsign

  - Frequency: Match what you’ll program into the IC-9700 (e.g., 433.550 MHz)

  - Mode: Enable D-STAR

  - Duplex: Set to 0 (for simplex)

- Under D-STAR Configuration:

  - Set RPT1 Callsign: YOURCALL B (for UHF) or YOURCALL C (for VHF)

  - Set RPT2 Callsign: YOURCALL G (for gateway)

- Click Apply Changes

📻 Step 2: Program the IC-9700

Option A: Manual Entry

- Set the radio to DV mode

- Set the frequency to match your hotspot (e.g., 433.550 MHz)

- Set URCALL to CQCQCQ

- Set RPT1 to YOURCALL B (or C for VHF)

- Set RPT2 to YOURCALL G

- Save to a memory channel if desired

Option B: Using DR Mode

- Press the DR button

- Tap FROM > Repeater List > Add

- Enter:

  - Name: e.g., MyHotspot

  - Callsign: YOURCALL B

  - Frequency: 433.550 MHz

  - Offset: 0.000 MHz

  - Use Repeater Type: Simplex

- Tap TO > Use Reflector or CQCQCQ to talk

🧪 Test It Out

- Try an echo test by setting URCALL to ________E (7 spaces + E)

- Or link to a reflector with REF030CL in URCALL

# DR Mode

DR Mode simplifies D-STAR operation by letting you select repeaters and reflectors from a menu instead of manually entering callsigns.

🎛️ What Is DR Mode?

DR Mode (Digital Repeater Mode) is a user-friendly interface for D-STAR that allows you to:

- Easily select repeaters and reflectors

- Link/unlink from reflectors

- Perform echo tests and info queries

- Use GPS-based repeater search (if enabled)

🛠️ Step-by-Step: Using DR Mode on the IC-9700

🔹 Step 1: Enter DR Mode

- Press the DR button on the front panel.

- The screen will now show two fields:

  - TO (who or what you’re calling)

  - FROM (which repeater or hotspot you’re using)

🔹 Step 2: Set the FROM Field (Your Repeater or Hotspot)

- Tap the FROM field.

- Choose one of the following:

  - Repeater List: Select a preloaded repeater

  - Near Repeater: If GPS is enabled, shows nearby repeaters

  - Direct Input: Manually enter a repeater callsign

- Select your desired repeater (e.g., VA3ODC B)

💡 If your repeater isn’t listed, you can add it manually or import a CSV file via SD card.

🔹 Step 3: Set the TO Field (Your Command or Destination)

- Tap the TO field.

- Choose from:

  - Use Reflector → CQCQCQ (for normal QSOs)

  - Link to Reflector → Enter a reflector (e.g., XLX405D)

  - Unlink Reflector → Sends unlink command

  - Echo Test → Sends _______E to hear yourself

  - Information → Sends _______I to get link status

🧠 These options automatically format the URCALL field correctly—no need to enter it manually.

🔹 Step 4: Transmit the Command

- After selecting a command (like Link to Reflector), press PTT briefly to send it.

- You’ll hear a voice confirmation if successful.

- Then go back to TO > Use Reflector > CQCQCQ to talk.

🔹 Step 5: Save to Memory (Optional)

- Press and hold the MW (Memory Write) button to save your DR setup to a memory channel for quick access later.

# Direct Commands

How to Enter Special URCALL Commands on the IC-9700

🔹 Step 1: Switch to DV Mode

- Press the MODE button.

- Select DV (Digital Voice) mode.

🔹 Step 2: Access the URCALL Field

- Press the DR button to enter DR mode.

- Tap the UR field on the screen (top right).

- Tap Direct Input.

🔹 Step 3: Enter the Special Command

Use the on-screen keyboard to enter the command. All URCALL entries must be exactly 8 characters long. Use spaces to pad shorter commands so the last character is in the 8th position.

| Command | What to Enter in URCALL | Function |
| --- | --- | --- |
| Link to REF | REF030CL | Link to REF030 Module C |
| Link to XLX | XLX405DL | Link to XLX405 Module D |
| Unlink | _______U | 7 spaces + U |
| Info | _______I | 7 spaces + I |
| Echo Test | _______E | 7 spaces + E |

🧩 Tip: Use the space key on the on-screen keyboard to pad the command. The 8th character is where the command letter (L, U, I, E) must go.

🔹 Step 4: Set RPT1 and RPT2

Make sure these are set correctly:

- RPT1: Your local repeater + port (e.g., VA3ODC B)

- RPT2: Same repeater + G (e.g., VA3ODC G)

To check or change:

- Tap the FROM field.

- Choose your repeater from the list or enter it manually.

🔹 Step 5: Send the Command

- Briefly press PTT to transmit the command.

- You should hear a voice confirmation or see a message on screen.

🧪 After Linking

Once linked, change URCALL back to CQCQCQ to begin normal voice communication.

# Pictures

Here’s a complete guide to sending and receiving pictures over D-STAR. This feature is built into the radio and allows you to share images with other D-STAR users—great for emergency comms, SSTV-style fun, or just experimenting with digital modes.

🖼️ What You Need

- Icom IC-9700 with firmware v1.20 or later

- A microSD card (for storing images)

- Another D-STAR station with picture capability (e.g., another IC-9700 or compatible device)

- DV mode enabled and a simplex or repeater frequency set

📸 Step-by-Step: Sending a Picture

- Insert a microSD card into the IC-9700.

- Press MENU > tap Picture.

- Tap Select Picture:

  - Choose a preloaded image from the SD card (JPG format, 320x240 recommended).

- Tap Send Picture:

  - Choose DV Fast Data (for faster transfer) or DV Voice (slower, but allows voice).

- Press PTT to transmit the image.

📌 Tip: You can also take a photo with your phone, resize it to 320x240, and save it to the SD card as a .jpg.

🖥️ Receiving a Picture

- Ensure you're in DV mode and tuned to the same frequency as the sending station.

- When a picture is transmitted, the IC-9700 will automatically receive and display it.

- You can save the received image to your SD card for later viewing.

⚙️ Picture Mode Settings

- DV Fast Data: Sends images quickly but requires both stations to support it.

- DV Voice: Slower, but allows simultaneous voice communication.

- Auto RX Save: Enable this to automatically save received images to the SD card.

# Appendix:

## ODC Repeaters

Location; Call Sign; Band; Frequency; Offset; RPT1 (Repeater); RPT2 (Gateway); Notes. Chatham; VA3ODC-C; VHF; 145.100 MHz; –0.600; VA3ODC C; VA3ODC G; Linked to XLX405D. Chatham; VA3ODC-B; UHF; 443.350 MHz; +5.000; VA3ODC B; VA3ODC G; Multimode. Wallaceburg; VE3XII-B; UHF; 446.125 MHz; –5.000; VE3XII B; VE3XII G; Linked to XLX405D. Tilbury; VE3CRO-B; UHF; 444.425 MHz; +5.000; VE3CRO B; VE3CRO G; Linked to XLX405D. Bala; VA3APW-B; UHF; 442.850 MHz; +5.000; VA3APW B; VA3APW G; Linked to XLX405D. Ridgetown; VE3IJC-B; UHF; 442.975 MHz; +5.000; VE3IJC B; VE3IJC G; Linked to XLX405D (pending). Leamington; VE3ESP-C; VHF; 147.300 MHz; +0.600; VE3ESP C; VE3ESP G; Linked to XLX405D (pending).

# D-STAR Special URCALL Commands Cheat Sheet

| URCALL | Function | Format Notes |
| --- | --- | --- |
| CQCQCQ | Standard voice call | Use this after linking to talk |
| REFxxxCL | Link to a reflector module (e.g., REF030C) | Replace xxx with reflector number |
| XLX405DL | Link to XLX reflector module D | Use for XLX reflectors (e.g., XLX405D) |
| _______U | Unlink from current reflector | 7 spaces + U (must be exactly 8 chars) |
| _______I | Info request (get current link status) | 7 spaces + I |
| _______E | Echo test (hear your own transmission) | 7 spaces + E |
| CALLSIGN E | Echo test via a specific repeater | Replace CALLSIGN with repeater call |
| CALLSIGN L | Link to a specific repeater | Replace CALLSIGN with target repeater |

🧩 Spacing is critical: All URCALL entries must be exactly 8 characters long. Use spaces to pad shorter entries so the command letter is in the 8th position.